Analysis of UNC Link Data

1 million packets, from UNC main connection

Time needed: ~ 3 seconds

Study both “incoming” and “outgoing”

View 1 of UNC Link Data

Packet Arrival Times, i.e. “rates”

SiZer version of smooth histogram

Show UncLinkData2p1d1.mpg and UncLinkData2p1d2.mpg

Notes:

- Overall high rate

- SiZer and SiCon show many “statistically significant bursts”

- SiZer patterns similar (for in vs. out), suggesting strong correlation.

- More packets outgoing (see density height and time shift)

View 2 of UNC Link Data

Packet Sizes, in bytes

SiZer version of smooth histogram

Show UncLinkData2p2d1.ps

Nicer on log scale???

Show UncLinkData2p3d1.mpg and UncLinkData2p3d2.mpg

Notes:

- widely separated values

- some common sizes  (e.g. 40 bytes)

- min = 28,    max = 1500

- more data outgoing (bigger packets)

View 2 (cont.)

Summary:

Percentages for special sizes:

| Size | Incoming | Outgoing |
| --- | --- | --- |
| 28 | 0.01% | 0.02% |
| 40 | 37% | 25% |
| 1500 | 12% | 33% |

View 3 of UNC Link Data

Packet Sizes per unit time

SiZer:  averages over different time scales

Show UncLinkData2p5d1.mpg and UncLinkData2p5d2.mpg

Notes:

- Statistically significant changes at many scales

- Bursty behavior

- Outgoing larger than incoming

- No apparent correlation

Views 4 and 5 of UNC Link Data

Views of binned data,

10,000 bins	⇒	scale   m≈0.02 sec

&	~100 obs. per bin

Study both:

- Counts (i.e. # packets in bin)

- Packet Size Totals (sum’d over bin)

View 4 of UNC Link Data

Autocorrelations

Show UncLinkData2p11d1.ps, UncLinkData2p11d2.ps, UncLinkData2p12d1.ps and UncLinkData2p12d2.ps

Big Surprise:

Have “mixture” of (~10%) long range dep. and (~90%) “white noise”

Possible Reasons???

- Studying wrong scale?

R&W:  m> “packet round trip time”   ⇒

⇒   Fractional Gaussian Noise

- Mixture of session/data types?

- ???

View 4 of UNC Link Data (cont.)

Autocorrelations (cont.)

Notes:

- “exponential intuition” from y≈a⋅eb⋅x⇔logy≈b⋅x+loga

- exp. fits with φ≈0.98 suggest “unit root”, i. e. long range dependence

- “polynomial intuition” from y≈a⋅xb⇔logy≈b⋅logx+loga

- poly fits, with powers  ∈−.28,−.08, suggest Hurst parameters:  H∈(0.86,0.96)

View 5 of UNC Link Data

Marginal Distributions

Show UncLinkData2p13d1.ps, UncLinkData2p13d2.ps, UncLinkData2p14d1.ps and UncLinkData2p14d2.ps

Notes:

- All are “roughly” both normal and log-normal

- Bin counts more normal?

- Packet Sizes more lognormal?

- Sizes have “few very small values”?

View 6 of UNC Link Data

Autocorrelations across scales

Best case:  Packet Sizes, Incoming

Show UncLinkData2p22d1.ps

Coarser Scales    ⇒

⇒	overall more dependence

⇒	steeper at left

⇒	more variability

View 6 of UNC Link Data (cont.)

Summaries of parameters:

Show UncLinkData2p22d1.ps

R2  for Long Range Dependence:

- “low” for “small” m∈(10−3,10−2)

- “increases” for m∈(10−2,10−1)

- “large” for m∈(10−1,1)

Power,   φ,   Hurst Parameter:

- Correlated

- “increasing for small scales m

View 6 of UNC Link Data (cont.)

Other cases  (outgoing, Bin Counts)

- somewhat similar

- more “noise” problems???

Worst case:  Outgoing Packet Sizes

Show UncLinkData2p22d2.mpg and UncLinkData2p22d2.ps

- Fit lines sometimes slope down

- Then φ>1   ?!?

- and Hurst Param. > 1  ?!?

Should Pursue Further?

- Study autocorr’s at coarses scales m?  (needs more data)

- Modify simulations, to show observed autocorr. Structure?

- Careful look at sessions (to explain autocorr.)?

- Other explanations of autocorr.?

5.	Investigate “few small sizes”?